UKMARSBOT I2C Sensor Controller Library Documentation

# Table of Contents

1.0	LIBRARY OVERVIEW	1

2.0	INSTALLATION	1

3.0	FUNCTION DESCRIPTIONS	2

4.0	EXAMPLES	16

5.0	Application Note – Custom Sensor Board Configuration:	19

The ISC has a supporting Arduino library to simplify the integration. This document provides an overview of the Arduino ISC library, describing the functions, their parameters and return values along with examples. Finally, example sketches are provided to give practical examples of how the device may be used.

It is recommended, for better understanding of the capabilities of the device, that the ISC Hardware Datasheet be read in conjunction with this document.

# LIBRARY OVERVIEW

The below diagram indicates schematically how the functions within the Arduino ISC Library may be used in conjunction with an ISC board.

# INSTALLATION

- The ISC library and all other supporting files are provided at the following link:

https://github.com/slinkyfish/ISC-Project

- Download the project as a compressed file.

- Extract the downloaded file and copy the ISC folder into the libraries folder within the Arduino installation. The directory structure should look like the below image.

- Once the Arduino software is restarted, the library will be available to include, and example code detailed in Section 4.0 will be available in the Examples menu.

- To use this library in any sketch, include the following at the start of the code:

#include <ISC.h>

# FUNCTION DESCRIPTIONS

The functions included in the Arduino ISC Library are shown schematically in the graphic below, then listed with brief description and link to detailed description after.

Global

reset()		//Forces the ISC to a reset state, starting initial setup

begin()		//Starts the ISC in run mode, defines sensor board

scanTime()		//Returns the time taken for the ISC to complete a full cycle

LED Indicators

ledMode()		//Controls the Indicator LED function

setLeftLED()	//Direct control of Left LED state

setRightLED()	//Direct control of Right LED state

Individual sensors

configure()		//Individual sensor setup configuration

setThreshold()	//Specifies the value, above which, the state is ON

getVal()		//Returns the value of specified sensor

Overview of sensors

states()		//Returns a byte, with bits representing sensor states

txPulse()		//Sets time that Sensor Tx. is on before first sample is taken

Interrupts

setInterrupts()	//Set components that can generate an interrupt

getInterrupts()	//Determine which component(s) caused the interrupt

clearInterrupts()	//Acknowledges interrupts, clearing them down for next time

## reset()

Description

Forces the ISC to a reset state, allowing the initial setup to be performed.

Syntax

void reset(uint8_t address);

Parameters

address	7-bit I2C address for ISC device – printed on device.

Returns

N/A

Example

Isc.reset(0x50); //Reset ISC with I2C (7-bit) Address of 0x50

## begin()

Description

This function starts the ISC in run mode, confirming that setup is complete. It defines the sensor board that is attached.

Syntax

void begin(uint8_t boardType);

Parameters

boardType	Type of sensor board connected to the ISC:

basicLineSensor		UKMARS Basic Line Sensor Board

basicWallSensor		UKMARS Basic Wall follower Board

spLineSensor		Line Sensor board by S. Pithouse

customBoard			Undefined board

Returns

N/A

Example

Isc.begin(basicLineSensor); //Start with Basic Line sensor attached

## scanTime()

Description

This function returns the most recent time taken for the ISC to complete a full cycle. The units are µs.

Syntax

int scanTime();

Parameters

N/A

Returns

Most recent time taken for the ISC to complete a full cycle (µs).

Example

int myTime = Isc.scanTime(); //Store most recent scan time in myTime

## setThreshold()

Description

This function allows the sensor threshold to be set for a specific sensor.

Syntax

void setThreshold(uint8_t sens, uint16_t threshVal);

Parameters

Sensor

SENSOR0	See diagram in Section ?? – A0 on Sensor Board

SENSOR1	See diagram in Section ?? – A0 on Sensor Board

SENSOR2 	See diagram in Section ?? – A0 on Sensor Board

SENSOR3 	See diagram in Section ?? – A0 on Sensor Board

SENSOR4 	See diagram in Section ?? – A0 on Sensor Board

threshVal

Value between 0 – 1024

Returns

N/A

Example

Isc.setThreshold(SENSOR0, 500);

## getVal()

Description

This function returns the most recent sensor value for the specified sensor.

Syntax

int getVal(uint8_t sensor);

Parameters

Sensor

SENSOR0	See diagram in Section ?? – ‘A0’ on Sensor Board

SENSOR1	See diagram in Section ?? – ‘A1’ on Sensor Board

SENSOR2 	See diagram in Section ?? – ‘A2’ on Sensor Board

SENSOR3 	See diagram in Section ?? – ‘A3’ on Sensor Board

SENSOR4 	See diagram in Section ?? – ‘A4’ on Sensor Board

Returns

Value of specified sensor

Example

leftSensor = Isc.getVal(SENSOR0);

## ledMode()

Description

This function controls the Indicator LED function

Syntax

void ledMode(uint8_t mode);

Parameters

mode

MASTERCTRL		MCU has control

FREQOUT		Scan frequency output on LEDs

LEDOFF		Turn LEDs off

LEDBRDCTRL		Allow board specific LED control

FASTBLINK		Start LEDS blinking fast

SLOWBLINK		Start LEDs blinking slowly

Returns

N/A

Example

Isc.ledMode(FASTBLINK);

## setLeftLED()

Description

This function controls the Indicator LED function. Reads current led state, sets into Master control mode and turns Left LED on.

Syntax

void setLeftLED(uint8_t state);

Parameters

state

HIGH		Turn LED on

LOW		Turn LED off

Returns

N/A

Example

Isc.setLeftLED(HIGH);

## setRightLED()

Description

This function controls the Indicator LED function. Reads current led state, sets into Master control mode and turns Left LED on.

Syntax

void setRightLED(uint8_t state);

Parameters

state

HIGH		Turn LED on

LOW		Turn LED off

Returns

N/A

Example

Isc.setRightLED(HIGH);

## configure()

Description

This function allows individual sensor setup to be configured. There are keywords that may be combined with ‘+’ as shown.

Syntax

Isc.configure(uint8_t sensor, uint8_t config);

Parameters

sensor

SENSOR0	See diagram in Section ?? – ‘A0’ on Sensor Board

SENSOR1	See diagram in Section ?? – ‘A1’ on Sensor Board

SENSOR2 	See diagram in Section ?? – ‘A2’ on Sensor Board

SENSOR3 	See diagram in Section ?? – ‘A3’ on Sensor Board

SENSOR4 	See diagram in Section ?? – ‘A4’ on Sensor Board

config

enb			Enable sensor – ISC will read the value

lowRes		Reduce value to 8-bit

txEnb			Enable Transmitter to help remove ambient light

fallingInterrupt		Sensor will cause interrupt on 1->0 state change

risingInterrupt		Sensor will cause interrupt on 0->1 state change

flip			[See Polarity]

Returns

N/A

Example

Isc.configure(SENSOR0, enb + txEnb + risingInterrupt); //Marker Sensor

## txPulse()

Description

This function allows the time in µs that the Tx emitter is on before the first sample is taken to be adjusted.

Syntax

void txPulse(uint8_t length);

Parameters

Length

Amount of time in us…??

Returns

N/A

Example

Isc.txPulse(100);

## states()

Description

This function gets the current states of the sensors and returns a byte with 1 representing HIGH sensor state.

Syntax

uint8_t states();

Parameters

N/A

Returns

Byte with bits indicating current sensor state

Example

leftState = Isc.states() & 0b1;

## read()

Description

Reads a byte (or pair of bytes) from a specified register within the ISC.

Syntax

int read(uint8_t regAddr, uint16_t numVals);

Parameters

regAddr

Register address to read from

numVals

May be 1 or 2, for the number of consecutive registers to read (ie. 2 when a 2-byte

value is to be read).

Returns

The value stored in that location (int)

Example

int timeTaken = Isc.read(SCANTIME, 1);//Read 1 byte at address SCANTIME

## write()

Description

Writes a byte (or pair of bytes) to a specified register within the ISC.

Syntax

void write(uint8_t regAddr, uint16_t data, uint8_t numVals);

Parameters

regAddr

Register address to write to

data

Data to write

numVals

May be 1 or 2, for the number of consecutive registers to write (ie. 2 when a 2-byte

value is to be written).

Returns

N/A

Example

Isc.write(SENS0THRSH, 500, 2);//Write 500 (2 bytes) for Sensor 0 Threshold

## setInterrupts()

Description

Set components that can generate an interrupt.

Syntax

void setInterrupts(uint8_t interrupts);

Parameters

interrupts

SENSOR0INTERRUPT		Interrupt according to Sensor 0 Configuration

SENSOR1INTERRUPT		Interrupt according to Sensor 1 Configuration

SENSOR2INTERRUPT		Interrupt according to Sensor 2 Configuration SENSOR3INTERRUPT		Interrupt according to Sensor 3 Configuration

SENSOR4INTERRUPT		Interrupt according to Sensor 4 Configuration

Returns

N/A

Example

setInterrupts(SENSOR0INTERRUPT + SENSOR1INTERRUPT); //Interrupt on Sensor 0 and Sensor 1 as per their configuration (rising/falling)

## getInterrupts()

Description

Reads a byte (or pair of bytes) from a specified register within the ISC.

Syntax

uint8_t getInterrupts();

Parameters

N/A

Returns

Active Interrupts which may be interrogated

Example

int activeInterrupts = getInterrupts(); //Store components which triggered interrupt

## clearInterrupts()

Description

Acknowledges interrupts, clearing them down for next time.

Syntax

void clearInterrupts(uint8_t interrupts);

Parameters

interrupts

SENSOR0INTERRUPT		Interrupt according to Sensor 0 Configuration

SENSOR1INTERRUPT		Interrupt according to Sensor 1 Configuration

SENSOR2INTERRUPT		Interrupt according to Sensor 2 Configuration SENSOR3INTERRUPT		Interrupt according to Sensor 3 Configuration

SENSOR4INTERRUPT		Interrupt according to Sensor 4 Configuration

Returns

N/A

Example

clearInterrupts(SENSOR0INTERRUPT + SENSOR1INTERRUPT); //Clear down Sensor 0 and Sensor 1 Interrupt flags so they can trigger again

# EXAMPLES

## Example_0_Blink

Initialise the ISC and set LED mode to SLOWBLINK.

#include <ISC.h>

ISC Isc;

void setup(){

Isc.reset(0x50);             //Establish communication and Reset ISC (Address 0x50)

Isc.begin(basicLineSensor);

ledMode(SLOWBLINK);

}

void loop(){

}

## Example_1_LeftLED

Initialise the ISC and set Left LED on.

#include <ISC.h>

ISC Isc;

void setup(){

Isc.reset(0x50);             //Establish communication and Reset ISC (Address 0x50)

Isc.begin(basicLineSensor);

leftLED(HIGH);

}

void loop(){

}

## Example_2_ReadSensor

Initialise the ISC and read sensor 0 value every 100ms.

#include <ISC.h>

ISC Isc;

void setup(){

Serial.begin(115200);		//Start communications over Serial Port

Isc.reset(0x50);             //Establish communication and Reset ISC (Address 0x50)

Isc.setup(SENSOR0, enb+txEnb);   //Setup Left Sensor, enabled with Tx enabled

Isc.txPulse(10);                 //10µs Tx on time before sampling begins

Isc.begin(basicLineSensor);

}

void loop(){

Serial.println(Isc.getVal(SENSOR0));

delay(100);

}

## Example_3_ScanTime

Initialise the ISC and report the scan time every 100ms

#include <ISC.h>

ISC Isc;

void setup(){

Serial.begin(115200);		//Start communications over Serial Port

Isc.reset(0x50);             //Establish communication and Reset ISC (Address 0x50)

Isc.begin(basicLineSensor);

}

void loop(){

Serial.println(Isc.scanTime(SENSOR0));

delay(100);

}

## Example_4_Interrupts

Configure a sensor increment a count each time it exceeds a boundary

# TEXT – KEYWORDS

Isc.configure(SENSOR0, enb + txEnb + risingInterrupt);     //Configure Marker Sensor

# FUNCTIONS

editor.keyword1.style = #d35400,bold

editor.data_type.style = #d35400,bold

# METHODS

editor.keyword2.style = #D35400,plain

editor.function.style = #d35400,plain

# STRUCTURES

editor.keyword3.style = #5E6D03,plain

editor.reserved_word.style = #5E6D03,plain

# TEXT - LITERALS

editor.literal1.style = #00979C,plain

editor.literal2.style = #00979C,plain

editor.variable.style = #00979C,plain

editor.reserved_word_2.style = #00979C,plain

editor.literal_boolean.style = #00979C,plain

editor.literal_char.style = #00979C,plain

editor.literal_string_double_quote.style = #005C5F,plain

editor.preprocessor.style = #5E6D03,plain

# http://www.arduino.cc/

editor.url.style = #0000ff,underlined

# e.g. + - = /

editor.operator.style = #434f54,plain

# ?? maybe this is for words followed by a colon

# like in case statements or goto

editor.label.style = #7e7e7e,bold

# TEXT - COMMENTS

editor.comment1.style = #434F54,plain		dk grey /**/

editor.comment2.style = #95a5a6,plain		light grey //

# LINE STATUS - editor line number status bar at the bottom of the screen

linestatus.font	   = SansSerif,plain,10

linestatus.height  = 20

# Application Note – Custom Sensor Board Configuration:

Basic Line Sensor Connections